# Control Theory: Tutorial with Julia

Iacopo Moles

Table of contents

# 1. Introduction

## 1.1 Intro

This is a collection of write ups on how to solve the various problems presented by Github user “Janismac”.

## 1.2 What do I need?

### 1.2.1 Software

- A real OS like Linux or Windows.

- The Julia Programming Language

  - Clone the repo

  - Activate the package by running in your terminal:

  - julia --project -e 'using Pkg; Pkg.instantiate()'

- (Nice to have) OpenModelica Editor

### 1.2.2 Theory

- Basic Julia knowledge

- Basic JS knowledge

- Control Theory knowledge

  - Frequency Based Control

  - State Space Based control

- Misc knowledge:

  - Linear Algebra

  - Differential Equations

# 2. The Damped Mass problem

## 2.1 Modeling

To better understand the problem let’s take a peek at how the simulated model works.

Models.BlockOnSlope.prototype.vars = 
{
    g: 9.81,
    x: 0,          // distance from objective s
    dx: 0,         // velocity v
    slope: 1,      // slope coefficient alpha = dy/dx in the cartesian plane
    F: 0,          // Requested u
    F_cmd: 0,      // Saturated u
    friction: 0,   // Coulomb friction coefficient mu
    T: 0,          // Simulation Time
};

Models.BlockOnSlope.prototype.simulate = function (dt, controlFunc)
{
    this.F_cmd = controlFunc({x:this.x,dx:this.dx,T:this.T});
    if(typeof this.F_cmd != 'number' || isNaN(this.F_cmd)) throw "Error: The controlFunction must return a number.";
    this.F_cmd = Math.max(-20,Math.min(20,this.F_cmd));
    integrationStep(this, ['x', 'dx', 'F'], dt);
}

Models.BlockOnSlope.prototype.ode = function (x)
{
    return [
        x[1],
        (x[2]) - (Math.sin(this.slope) * this.g) - (this.friction * x[1]),
        20.0 * (this.F_cmd - x[2])
    ];
}

Line 2

The model has obviously some default values for the parameters that can be modified for the different scenarios.

Line 17

The control command u is generated by the controlFunction(block) function provided by us. There are some checks to see if it’s a number. If it’s acceptable then it passes through a saturation between ±20.

Lines 24-26

The model is a simple ODE with equations:

s=vv=F−sinα⋅g−μ⋅vF=−20⋅F+20⋅usat

Converting it in state-space representation:

svF=0100−μ100−20svF+00usat+0−sinα⋅g0

sv=100010svF

Obviously the gravitational term acts as a disturbance.

Converting it in transfer function form is not necessary.

α+

# 3. Block Without Friction

Position Control with friction. Using Pole Placement + PD.

## 3.1 State Space representation

We can convert the set of ODE into a state space representation. The final bode plot of the block position is:

using DiscretePIDs, ControlSystems, Plots, LinearAlgebra

# System parameters
Ts = 0.02 # sampling time
Tf = 2.5; #final simulation time
g = 9.81 #gravity
α = 0.0 # slope
μ = 1.0 # friction coefficient
x_0 = -2.0 # starting position
dx_0 = 0.0 # starting velocity
τ = 20.0 # torque constant 

# State Space Matrix
A = [0 1 0
    0 -μ 1
    0 0 -τ];
B = [0
    0
    τ];
C = [1 0 0
    0 1 0];

sys = ss(A, B, C, 0.0)      # Continuous

plot!(bodeplot(tf(sys)),pzmap(tf(sys)))

Starting Bode Plot and PZ Map

It has the shape we expect from a motor + friction. Slow pole for the mass + friction and a faster pole for the current & inductance.

Numerically they are:

display(eigvals(A))

3-element Vector{Float64}:
 -20.0
  -1.0
   0.0

We see that we start with all the pole in the left-half plane, which is good.

## 3.2 Pole Placement

We can design a controller with pole placement.

For some reason pole placement doesn’t work for the observer, I use a Kalman Filter with random fast values.

observability(A, C).isobservable & 
controllability(A, B).iscontrollable; #OK

ε = 0.01;
pp = 15.0;
poles_cont = -2.0 * [pp + ε, pp - ε, pp];
L = real(place(sys, poles_cont, :c));

poles_obs = poles_cont * 10.0;
K = place(1.0 * A', 1.0 * C', poles_obs)'
cont = observer_controller(sys, L, K; direct=false);

We can check the effect of the new controller on the loop

closedLoop = feedback(sys * cont)
print(poles(closedLoop));
setPlotScale("dB")
plot!(bodeplot(closedLoop[1, 1], 0.1:40), pzmap(closedLoop))

ComplexF64[-29.979998924597755 + 0.0im, -30.000002152199972 + 0.0im, -30.019998923202337 + 0.0im, -300.0000000000004 + 0.0im, -300.1999999999752 + 0.0im, -299.80000000004117 + 0.0im]

We can compare this to the open-loop response in @start-bode. We can see that we achieve unitary gain throughout the whole low-frequency range.

We can convert the pole placement controller into the standard PD gain form.

K = L[1];
Ti = 0;
Td = L[2] / L[1];
pid = DiscretePID(; K, Ts, Ti, Td);

## 3.3 Simulation

We can simulate this with a motor that only outputs the position:

sysreal = ss(A, B, [1 0 0], 0.0)
ctrl = function (x, t)
    y = (sysreal.C*x)[] # measurement
    d = 0 * [1.0]        # disturbance
    r = 2.0 * (t >= 1) # reference
    # u = pid(r, y) # control signal
    # u + d # Plant input is control signal + disturbance
    # u =1
    e = x - [r; 0.0; 0.0]
    e[3] = 0.0 # torque not observable, just ignore it in the final feedback
    u = -L * e + d
    u = [maximum([-20.0 minimum([20.0 u])])]
end
t = 0:Ts:Tf

res = lsim(sysreal, ctrl, t)

display(plot(res, 
    plotu=true, 
    plotx=true, 
    ploty=false
    ))
ylabel!("u", sp=1);
ylabel!("x", sp=2);
ylabel!("v", sp=3);
ylabel!("T", sp=4);

For more stats:

si = stepinfo(res);
plot(si);title!("Step Response")

We can also simulate it in a SIMULINK-like environment:

using FMI, DifferentialEquations
fmuPath = abspath(joinpath(@__DIR__,
  "..","..","..",
  "modelica",
  "ControlChallenges",
  "ControlChallenges.BlockOnSlope_Challenges.Examples.WithFriction.fmu"))
fmu = loadFMU(fmuPath);
simData = simulateME(
    fmu,
    (0.0, 5.0);
    recordValues=["blockOnSlope.x", 
    "blockOnSlope.xd", 
    "blockOnSlope.usat"],
    showProgress=false);
unloadFMU(fmu);
plot(simData, states=false, timeEvents=false)

There is a slight difference between the lsim simulation and the FMU simulation. I need to recheck some stuff.

let Kp = 337;
let Kd = 64;
function controlFunction(block)
{
  return -( block.x * Kp + block.dx*Kd);
}

Open Simulation

# 4. Block With Friction

Position Control with friction. Using Pole Placement + PD.

## 4.1 Response Analysis

using CCS
using ControlSystems, Plots, LinearAlgebra, RobustAndOptimalControl
CCS.setupEnv()

contSys = CCS.blockModel.csys(;g = 0, α = 0 , μ = 1, τ =20)
plot!(bodeplot(contSys[1,1]),pzmap(contSys))

Starting Bode Plot and PZ Map

It has the shape we expect from a motor + friction. Slow pole for the mass + friction and a faster pole for the current & inductance.

Numerically they are:

display(eigvals(contSys.A))

3-element Vector{Float64}:
 -20.0
  -1.0
   0.0

We see that we start with all the poles in the left-half plane, which is good.

## 4.2 Pole Placement

We can design a controller with pole placement.

For some reason pole placement doesn’t work for the observer, I use a Kalman Filter with random fast values.

observability(contSys.A,contSys.C).isobservable || error("System is not observable")
controllability(contSys.A,contSys.B).iscontrollable || error("System is not controllable")

ε = 0.01;
pp = 15.0;
poles_cont = - [pp + ε, pp - ε, pp];
L = real(place(contSys, poles_cont, :c));


poles_obs = poles_cont * 10.0;
K = place(contSys, poles_obs, :o)
obs_controller = observer_controller(contSys, L, K; direct=false);
fsf_controller = named_ss(obs_controller, u = [:ref_S, :ref_V], y = [:u]);

We can check the effect of the new controller on the loop

closedLoop = feedback( contSys * fsf_controller);
print(poles(closedLoop));
setPlotScale("dB")
plot!(bodeplot(closedLoop[1,1], 0.1:40), pzmap(closedLoop))

ComplexF64[-14.990000366343788 + 0.0im, -14.999999266673722 + 0.0im, -15.010000366982666 + 0.0im, -149.99999999999986 + 0.0im, -150.10000000002432 + 0.0im, -149.8999999999607 + 0.0im]

We can compare this to the open-loop response in @start-bode. We can see that we achieve unitary gain throughout the whole low-frequency range.

We can convert the pole placement controller into the standard PD gain form.

using DiscretePIDs
Ts = 0.02 # sampling time
Tf = 2.5; #final simulation time

K = L[1];
Ti = 0;
Td = L[2] / L[1];

pid = DiscretePID(; K, Ts, Ti, Td);

## 4.3 Simulation

We can simulate this with a motor that only outputs the position:

sysreal = ss(contSys.A, contSys.B, [1 0 0], 0.0)
ctrl = function (x, t)
    y = (sysreal.C*x)[] # measurement
    d = 0 * [1.0]        # disturbance
    r = 2.0 * (t >= 1) # reference
    # u = pid(r, y) # control signal
    # u + d # Plant input is control signal + disturbance
    # u =1
    e = x - [r; 0.0; 0.0]
    e[3] = 0.0 # torque not observable, just ignore it in the final feedback
    u = -L * e + d
    u = [maximum([-20.0 minimum([20.0 u])])]
end
t = 0:Ts:Tf

res = lsim(sysreal, ctrl, t)

display(plot(res, 
    plotu=true, 
    plotx=true, 
    ploty=false
    ))
ylabel!("u", sp=1);
ylabel!("x", sp=2);
ylabel!("v", sp=3);
ylabel!("T", sp=4);

For more stats:

si = stepinfo(res);
plot(si);title!("Step Response")

We can also simulate it in a SIMULINK-like environment:

using FMI, DifferentialEquations
fmuPath = abspath(joinpath(@__DIR__,
  "..","..","..",
  "modelica",
  "ControlChallenges",
  "ControlChallenges.BlockOnSlope_Challenges.Examples.WithFriction.fmu"))
fmu = loadFMU(fmuPath);
simData = simulateME(
    fmu,
    (0.0, 5.0);
    recordValues=["blockOnSlope.x", 
    "blockOnSlope.xd", 
    "blockOnSlope.usat"],
    showProgress=false);
unloadFMU(fmu);
plot(simData, states=false, timeEvents=false)

There is a slight difference between the lsim simulation and the FMU simulation. I need to recheck some stuff.

let Kp = 337;
let Kd = 64;
function controlFunction(block)
{
  return -( block.x * Kp + block.dx*Kd);
}

Open Simulation

# 5. Block on a slope

Position Control with friction. Using Pole Placement + PD.

## 5.1 Response Analysis

using CCS
using ControlSystems, Plots, LinearAlgebra, RobustAndOptimalControl
CCS.setupEnv()

contSys = CCS.blockModel.csys(;g = 0, α = 0 , μ = 1, τ =20)
plot!(bodeplot(contSys[1,1]),pzmap(contSys))

Starting Bode Plot and PZ Map

It has the shape we expect from a motor + friction. Slow pole for the mass + friction and a faster pole for the current & inductance.

Numerically they are:

display(eigvals(contSys.A))

3-element Vector{Float64}:
 -20.0
  -1.0
   0.0

We see that we start with all the poles in the left-half plane, which is good.

## 5.2 Pole Placement

We can design a controller with pole placement.

For some reason pole placement doesn’t work for the observer, I use a Kalman Filter with random fast values.

observability(contSys.A,contSys.C).isobservable || error("System is not observable")
controllability(contSys.A,contSys.B).iscontrollable || error("System is not controllable")

ε = 0.01;
pp = 15.0;
poles_cont = - [pp + ε, pp - ε, pp];
L = real(place(contSys, poles_cont, :c));


poles_obs = poles_cont * 10.0;
K = place(contSys, poles_obs, :o)
obs_controller = observer_controller(contSys, L, K; direct=false);
fsf_controller = named_ss(obs_controller, u = [:ref_S, :ref_V], y = [:u])

NamedStateSpace{Continuous, Float64}
A = 
  -150.0                         8.033684828490095e-12    0.0
     1.0915557686859107e-5    -279.9999999999851          1.0
 -3374.9970813429577        -17530.98989999806          -44.0
B = 
 150.0                        0.9999999999919663
  -1.0915557686859107e-5    278.9999999999851
  -0.0014186570429435138  16899.989999998063
C = 
 168.74992500000002  31.549995000000003  1.2000000000000002
D = 
 0.0  0.0

Continuous-time state-space model
With state  names: x1 x2 x3
     input  names: ref_S ref_V
     output names: u

We can check the effect of the new controller on the loop

closedLoop = feedback( contSys * fsf_controller);
print(poles(closedLoop));
setPlotScale("dB")
plot!(bodeplot(closedLoop[1,1], 0.1:40), pzmap(closedLoop))

ComplexF64[-14.990000366343788 + 0.0im, -14.999999266673722 + 0.0im, -15.010000366982666 + 0.0im, -149.99999999999986 + 0.0im, -150.10000000002432 + 0.0im, -149.8999999999607 + 0.0im]

We can compare this to the open-loop response in @start-bode. We can see that we achieve unitary gain throughout the whole low-frequency range.

We can convert the pole placement controller into the standard PD gain form.

using DiscretePIDs
Ts = 0.02 # sampling time
Tf = 2.5; #final simulation time

K = L[1];
Ti = 0;
Td = L[2] / L[1];

pid = DiscretePID(; K, Ts, Ti, Td);

## 5.3 Simulation

We can simulate this with a motor that only outputs the position:

sysreal = ss(contSys.A, contSys.B, [1 0 0], 0.0)
ctrl = function (x, t)
    y = (sysreal.C*x)[] # measurement
    d = 0 * [1.0]        # disturbance
    r = 2.0 * (t >= 1) # reference
    # u = pid(r, y) # control signal
    # u + d # Plant input is control signal + disturbance
    # u =1
    e = x - [r; 0.0; 0.0]
    e[3] = 0.0 # torque not observable, just ignore it in the final feedback
    u = -L * e + d
    u = [maximum([-20.0 minimum([20.0 u])])]
end
t = 0:Ts:Tf

res = lsim(sysreal, ctrl, t)

display(plot(res, 
    plotu=true, 
    plotx=true, 
    ploty=false
    ))
ylabel!("u", sp=1);
ylabel!("x", sp=2);
ylabel!("v", sp=3);
ylabel!("T", sp=4);

For more stats:

si = stepinfo(res);
plot(si);title!("Step Response")

We can also simulate it in a SIMULINK-like environment:

using FMI, DifferentialEquations
fmuPath = abspath(joinpath(@__DIR__,
  "..","..","..",
  "modelica",
  "ControlChallenges",
  "ControlChallenges.BlockOnSlope_Challenges.Examples.WithFriction.fmu"))
fmu = loadFMU(fmuPath);
simData = simulateME(
    fmu,
    (0.0, 5.0);
    recordValues=["blockOnSlope.x", 
    "blockOnSlope.xd", 
    "blockOnSlope.usat"],
    showProgress=false);
unloadFMU(fmu);
plot(simData, states=false, timeEvents=false)

There is a slight difference between the lsim simulation and the FMU simulation. I need to recheck some stuff.

let Kp = 337;
let Kd = 64;
function controlFunction(block)
{
  return -( block.x * Kp + block.dx*Kd);
}

Open Simulation

# Appendix A — Performance tricks

## A.1 Hurwitz Check

Create our nice model. Assume to have run the poles function and that you have a vector of eigenvalues. For simplicity I will create an arbitrary vector with the first 100 values in −1 and the last 100 values as random around 0.

using BenchmarkTools

vbig = [zeros(ComplexF64,100).-1 ; rand(ComplexF64,100).-0.5];

vbig[[1,end]]

2-element Vector{ComplexF64}:
                 -1.0 + 0.0im
 -0.24799024016810423 + 0.8662500972954771im

### A.1.1 for loop

With a naive approach we check if all the elements are in the LHP: make a function that iterates and returns false if it hits a pole with positive real part.

function isHurwitz(v)
    for i in eachindex(v)
        if real(v[i])>0.0
            return false
        end
    end
    return true
end

@benchmark isHurwitz($(Ref(vbig))[])

BenchmarkTools.Trial: 10000 samples with 987 evaluations per sample.
 Range (min … max):  50.659 ns … 360.588 ns  ┊ GC (min … max): 0.00% … 0.00%
 Time  (median):     52.280 ns               ┊ GC (median):    0.00%
 Time  (mean ± σ):   53.140 ns ±   7.422 ns  ┊ GC (mean ± σ):  0.00% ± 0.00%

  █▆▆▃▄ █ ▅▁▁▆  ▁▁▁                                            ▂
  █████▇███████▇███▆▆▇▆▆▄▄▅▄▃▄▄▄▄▁▅▁▄▃▁▃▄▁▄▄▁▁▃▃▃▄▃▃▃▁▁▁▃▄▁▄▁▃ █
  50.7 ns       Histogram: log(frequency) by time      73.2 ns <

 Memory estimate: 0 bytes, allocs estimate: 0.

We have our baseline. We can probably squeeze out some more performance but I’m still a Julia noob.

### A.1.2 all()

Let’s try using some of the built-in declarative functions:

@benchmark all(real($vbig).<=0.0)

BenchmarkTools.Trial: 10000 samples with 776 evaluations per sample.
 Range (min … max):  170.876 ns …  18.307 μs  ┊ GC (min … max):  0.00% … 96.10%
 Time  (median):     238.660 ns               ┊ GC (median):     0.00%
 Time  (mean ± σ):   411.172 ns ± 851.020 ns  ┊ GC (mean ± σ):  36.72% ± 17.16%

  █▆▂  ▁▁                                                       ▁
  ████▇██▆▆▁▃▃▁▃▁▁▁▃▁▁▁▁▆▆▆▅▃▃▃▁▁▃▁▃▃▅▆▆▆▆▇▇▆▅▅▅▄▅▅▅▅▅▄▅▅▅▅▄▆▆▆ █
  171 ns        Histogram: log(frequency) by time       5.54 μs <

 Memory estimate: 1.75 KiB, allocs estimate: 5.

Simple all(), when given a tuple it checks if all the values are True, otherwise it stops when it encounters the first False.

We can see that it’s a tad slower. This is because it’s creating a new vector with just the real parts, then it’s creating a new vector with only the boolean results and then it’s checking if there are any False results.

This results in a lot of allocations and wasted resources.

@benchmark all(<=(0.0),real($vbig))

BenchmarkTools.Trial: 10000 samples with 796 evaluations per sample.
 Range (min … max):  150.000 ns …  24.051 μs  ┊ GC (min … max):  0.00% … 98.68%
 Time  (median):     215.327 ns               ┊ GC (median):     0.00%
 Time  (mean ± σ):   310.133 ns ± 551.533 ns  ┊ GC (mean ± σ):  22.76% ± 15.72%

  ▂█▇▄▂         ▃▁                                           ▁▁ ▂
  █████▇▇▆▅▅▃▃▃▇███▆▆▅▆▅▆▅▅▄▅▅▄▁▁▃▁▁▁▃▁▁▁▁▁▃▁▁▁▁▁▁▁▁▁▁▁▃▁▄▅▇███ █
  150 ns        Histogram: log(frequency) by time       2.14 μs <

 Memory estimate: 1.62 KiB, allocs estimate: 2.

A smarter way is to skip on of the allocations by creating the vector of real parts and then checking row by row if the non-positivity check fails.

@benchmark all(i -> real(i)<=0.0,$vbig)

BenchmarkTools.Trial: 10000 samples with 987 evaluations per sample.
 Range (min … max):  49.544 ns … 219.656 ns  ┊ GC (min … max): 0.00% … 0.00%
 Time  (median):     51.570 ns               ┊ GC (median):    0.00%
 Time  (mean ± σ):   53.682 ns ±  11.983 ns  ┊ GC (mean ± σ):  0.00% ± 0.00%

  ▁█▆▂▃▂                                                    ▁  ▁
  ███████▆▇▆▅▅▄▄▇▅▅▅▅▆▄▅▅▄▁▄▄▄▄▃▆▄▄▄▄▃▅▃▄▁▄▃▃▅▃▃▁▁▄▁▄▃▃▁▃▄▅▁██ █
  49.5 ns       Histogram: log(frequency) by time       116 ns <

 Memory estimate: 0 bytes, allocs estimate: 0.

We can do better: Instead of converting into real the full vector it checks element by element if it’s in the LHP. It returns false at the first failure. We finally have a comparable result to the benchmark function but in a more compact way.

Is it cleaner? That’s subjective.

### A.1.3 mapreduce()

Finally we try the MapReduce approach. This allows a better utilization of your processor without the necessity of learning parallel programming.

@benchmark mapreduce(i->real(i)<=0.0, &, $vbig)

BenchmarkTools.Trial: 10000 samples with 992 evaluations per sample.
 Range (min … max):  42.036 ns … 633.871 ns  ┊ GC (min … max): 0.00% … 0.00%
 Time  (median):     42.339 ns               ┊ GC (median):    0.00%
 Time  (mean ± σ):   43.741 ns ±  10.518 ns  ┊ GC (mean ± σ):  0.00% ± 0.00%

  █▄▁▂▂                                                        ▁
  █████▇█▆▆▆▆▅▇▄▄▁▇▇▄▃▄▅▃▃▁▁▁▄▁▃▁▁▁▃▁▁▁▄▁▁▁▃▁▁▃▁▁▃▁▁▁▁▁▃▃▃▁▃▃█ █
  42 ns         Histogram: log(frequency) by time      95.4 ns <

 Memory estimate: 0 bytes, allocs estimate: 0.

We squeeze the last bit of performance and beat the initial benchmark, not by much but still appreciable.

# Appendix B — Test OM.jl

Position Control with friction. Using Pole Placement + PD.

## B.1 Response Analysis

# import Pkg; Pkg.add("OMJulia")
using OMJulia

using OMJulia.API: API

omc = OMJulia.OMCSession();

[ Info: Path to zmq file="/tmp/openmodelica.icpmoles.port.julia.z0WV3KM0Lj"

modelfile = joinpath(@__DIR__,
  "modelica",
  "ControlChallenges",
  "package.mo")
API.loadFile(omc, modelfile)

classname = "ControlChallenges.BlockOnSlope_Challenges.Examples.WithFriction"
ModelicaSystem(omc,modelfile, classname)

fmuPath = API.buildModelFMU(omc, classname)

"/tmp/jl_Bdmswi/ControlChallenges.BlockOnSlope_Challenges.Examples.WithFriction.fmu"

using FMI, DifferentialEquations, Plots

fmu = loadFMU(fmuPath);
simData = simulateME(
    fmu,
    (0.0, 5.0);
    recordValues=["blockOnSlope.x", 
    "blockOnSlope.xd", 
    "blockOnSlope.usat"],
    showProgress=false);
unloadFMU(fmu);
plot(simData, states=false, timeEvents=false)

MacOs should be supported in theory but it’s not tested.